IMC Memory Block

SV File Reference

Complete port listings, internal signals, and inter-block connections

This document covers all four SystemVerilog files in the memory_block project:

| File | Module | Role |
| --- | --- | --- |
| reram_array.sv | reram_array | 16×16 ReRAM crossbar — stores weights and performs MAC |
| sram_block.sv | sram_block | 128KB SRAM — staging memory for input and output matrices |
| imc_peripheral.sv | imc_peripheral | AXI4-Lite slave + FSM — orchestrates ReRAM and DMA |
| imc_memory_block.sv | imc_memory_block | Top-level wrapper — wires all three modules together |

The testbench file tb_imc_memory_block.sv is the verification environment — it is not part of the synthesisable design.

# Block 1 — reram_array.sv

## 1.1  What It Does

Models a 16×16 ReRAM crossbar array. Each cell W[row][col] stores a 32-bit signed weight value. When given an input vector of 16 values, it multiplies the vector against the weight matrix and produces a 16-value output vector. All 16 dot products are computed simultaneously in a single 2-cycle pipeline.

Three operations are supported, selected by the mode input:

| mode[1:0] | Operation | What happens |
| --- | --- | --- |
| 2'b00 | IDLE | Nothing — all outputs hold their current values |
| 2'b01 | PROG | W[prog_row][prog_col] <= prog_weight (write one cell) |
| 2'b10 | COMPUTE | output_vec = input_vec × W (full MAC, 2-cycle latency) |
| 2'b11 | RESET | All W[i][j] cleared to zero |

## 1.2  Input Ports

| Port | Width | From | Description |
| --- | --- | --- | --- |
| clk | 1 | System | Clock — all registers on posedge |
| rst | 1 | System | Synchronous reset — clears W and in_buf |
| mode | 2 | imc_peripheral | Operation select: 00=idle 01=prog 10=compute 11=reset |
| prog_row | 4 | imc_peripheral | Row index (0–15) of cell to program |
| prog_col | 4 | imc_peripheral | Col index (0–15) of cell to program |
| prog_weight | 32 sgn | imc_peripheral | Weight value to write into W[prog_row][prog_col] |
| input_vec | 32×16 sgn | DMA | 16-element input vector — latched on mode=COMPUTE cycle |

## 1.3  Output Ports

| Port | Width | To | Description |
| --- | --- | --- | --- |
| output_vec | 32×16 sgn | DMA | Result of input_vec × W — valid 2 cycles after COMPUTE |
| done | 1 | imc_peripheral | Pulses HIGH for exactly 1 cycle when output_vec is ready |

## 1.4  Internal Signals (not visible outside)

| Signal | Width | Purpose |
| --- | --- | --- |
| W[16][16] | 32×16×16 sgn | The weight grid — persists until reset or overwritten. Models ReRAM non-volatility |
| in_buf[16] | 32×16 sgn | Snapshot of input_vec latched on the COMPUTE cycle |
| acc[16] | 64×16 sgn | 64-bit accumulators — one per output column, prevents overflow during MAC |
| sat[16] | 32×16 sgn | Saturated 32-bit results — clamped to INT32_MIN / INT32_MAX |
| delay | 1 | 1-cycle pipeline flag — goes HIGH the cycle after COMPUTE was seen, triggers output capture |

## 1.5  Timing

Cycle 0:  mode = COMPUTE  →  input_vec latched into in_buf

delay <= 1

Cycle 1:  delay = 1       →  MAC completes, sat[j] = input × W

output_vec registered, done = 1

Cycle 2:  done = 0        →  output_vec holds until next COMPUTE

⚠  mode must only be held HIGH for 1 cycle. The peripheral handles this — it pulses mode and returns to 2'b00 the next cycle.

## 1.6  MAC Equation

for each output column j (0 to 15):

output_vec[j] = SUM over i (in_buf[i] * W[i][j])

clamped to signed 32-bit range

⚠  Accumulator is 64-bit internally to prevent overflow during summation. Result is saturated, not truncated.

# Block 2 — sram_block.sv

## 2.1  What It Does

128KB synchronous SRAM organised as 32,768 words of 32 bits each. Provides two independent ports — Port A for the DMA (read and write) and Port B reserved for future use (read only, currently tied off in the top level). All reads have 1-cycle latency. No reset — contents persist until explicitly overwritten.

| Property | Value |
| --- | --- |
| Depth | 32,768 words |
| Width | 32 bits per word |
| Total size | 128 KB |
| Addressing | Word-addressed — addr 0 = first 32-bit word |
| Read latency | 1 cycle — address cycle N, data valid cycle N+1 |
| Write | Synchronous on posedge clk when a_we = 1 |
| Reset | None — contents persist across rst |

## 2.2  Input Ports — Port A (DMA)

| Port | Width | From | Description |
| --- | --- | --- | --- |
| clk | 1 | System | Clock |
| a_addr | 15 | DMA | Word address to read or write (0–32767) |
| a_wdata | 32 sgn | DMA | Data to write when a_we = 1 |
| a_we | 1 | DMA | Write enable — HIGH = write, LOW = read |

## 2.3  Output Ports — Port A (DMA)

| Port | Width | To | Description |
| --- | --- | --- | --- |
| a_rdata | 32 sgn | DMA | Read data — valid 1 cycle after a_addr presented |

## 2.4  Port B (Reserved — currently tied off)

| Port | Width | Current connection | Description |
| --- | --- | --- | --- |
| b_addr | 15 | 15'b0 (tied off) | Read address — unused |
| b_wdata | 32 sgn | 32'b0 (tied off) | Kept for interface compatibility — ignored |
| b_we | 1 | 1'b0 (tied off) | Kept for interface compatibility — ignored |
| b_rdata | 32 sgn | unconnected | Reserved output — available for future use |

## 2.5  Internal Signals

| Signal | Width | Purpose |
| --- | --- | --- |
| mem[32768] | 32×32768 sgn | The memory array — only Port A writes to it, single always_ff owner |

⚠  Port B is read-only by design — mem is written by exactly one always_ff block (Port A). This avoids multi-driver errors in Questa.

# Block 3 — imc_peripheral.sv

## 3.1  What It Does

The brain of the system. Contains two layers: an AXI4-Lite slave interface that accepts commands from the RISC-V core, and a 9-state FSM that orchestrates the sequence of operations across the ReRAM and DMA. It never touches SRAM directly — it only tells the DMA when and where to move data.

## 3.2  AXI4-Lite Input Ports (from RISC-V core)

| Port | Width | Channel | Description |
| --- | --- | --- | --- |
| clk | 1 | — | Clock |
| rst | 1 | — | Synchronous active-high reset |
| s_axi_awaddr | 32 | AW | Write address from core |
| s_axi_awvalid | 1 | AW | Core presenting a valid write address |
| s_axi_wdata | 32 | W | Write data from core |
| s_axi_wstrb | 4 | W | Byte strobes — accepted but ignored (always full 32-bit write) |
| s_axi_wvalid | 1 | W | Core presenting valid write data |
| s_axi_bready | 1 | B | Core ready to accept write response |
| s_axi_araddr | 32 | AR | Read address from core |
| s_axi_arvalid | 1 | AR | Core presenting a valid read address |
| s_axi_rready | 1 | R | Core ready to accept read data |

## 3.3  AXI4-Lite Output Ports (to RISC-V core)

| Port | Width | Channel | Description |
| --- | --- | --- | --- |
| s_axi_awready | 1 | AW | Peripheral ready to accept write address |
| s_axi_wready | 1 | W | Peripheral ready to accept write data |
| s_axi_bresp | 2 | B | Write response — always 2'b00 (OKAY) |
| s_axi_bvalid | 1 | B | Write response valid |
| s_axi_arready | 1 | AR | Peripheral ready to accept read address |
| s_axi_rdata | 32 | R | Read data — only STATUS register is readable |
| s_axi_rresp | 2 | R | Read response — always 2'b00 (OKAY) |
| s_axi_rvalid | 1 | R | Read data valid |

## 3.4  ReRAM Interface Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| reram_mode | 2 | OUT → reram_array | Operation pulse: 01=prog 10=compute 11=reset — 1 cycle only |
| reram_prog_row | 4 | OUT → reram_array | Row to write during PROG |
| reram_prog_col | 4 | OUT → reram_array | Col to write during PROG |
| reram_prog_weight | 32 sgn | OUT → reram_array | Weight value to program |
| reram_done | 1 | IN ← reram_array | 1-cycle pulse from ReRAM when compute is complete |

## 3.5  DMA Interface Ports

| Port | Width | Direction | Description |
| --- | --- | --- | --- |
| dma_trig_in | 1 | OUT → DMA | 1-cycle pulse — tells DMA to load next input row into ReRAM |
| dma_trig_out | 1 | OUT → DMA | 1-cycle pulse — tells DMA to save output row to SRAM |
| dma_src_addr | 15 | OUT → DMA | Base SRAM word address of matrix A |
| dma_dst_addr | 15 | OUT → DMA | Base SRAM word address for result matrix |
| dma_row | 4 | OUT → DMA | Current row index being processed (0–15) |
| dma_done | 1 | IN ← DMA | 1-cycle pulse — DMA signals it has finished its transfer |

## 3.6  Register Map

Core writes to BASE_ADDR + offset using standard SW instructions. Only STATUS is readable.

| Offset | Name | R/W | Description |
| --- | --- | --- | --- |
| 0x00 | PROG_ROW | W | Target row for weight programming (0–15) |
| 0x04 | PROG_COL | W | Target col for weight programming (0–15) |
| 0x08 | PROG_WEIGHT | W | 32-bit signed weight value to write |
| 0x0C | SRC_ADDR | W | SRAM word address where matrix A row 0 starts |
| 0x10 | DST_ADDR | W | SRAM word address where result row 0 will be written |
| 0x14 | TRIG_PROG | W | Write anything → programs one ReRAM cell (W[ROW][COL] = WEIGHT) |
| 0x18 | TRIG_COMPUTE | W | Write anything → starts full 16×16 matrix multiply |
| 0x1C | TRIG_RESET | W | Write anything → clears all weights to zero |
| 0x20 | STATUS | R | 0=idle 1=busy 2=done 3=error |
| 0x24 | CLEAR | W | Write anything → resets STATUS to 0 (idle) |

## 3.7  FSM States

| State | What it does | Exit condition |
| --- | --- | --- |
| S_IDLE | Waits idle. STATUS = 0 | Core writes TRIG_PROG / TRIG_COMPUTE / TRIG_RESET |
| S_PROG | Pulses reram_mode=PROG for 1 cycle. STATUS = 2 | Always → S_PROG_HOLD next cycle |
| S_PROG_HOLD | Holds STATUS=done, does nothing | Core writes CLEAR → S_IDLE |
| S_RESET | Pulses reram_mode=RESET for 1 cycle. STATUS = 2 | Always → S_RESET_HOLD next cycle |
| S_RESET_HOLD | Holds STATUS=done, does nothing | Core writes CLEAR → S_IDLE |
| S_WAIT_DMA_IN | Waits for DMA to finish loading input row | dma_done pulse → S_COMPUTE |
| S_COMPUTE | Pulses reram_mode=COMPUTE. Waits for ReRAM | reram_done pulse → S_WAIT_DMA_OUT |
| S_WAIT_DMA_OUT | Waits for DMA to finish saving output row | dma_done + row<15 → next row. dma_done + row=15 → S_DONE |
| S_DONE | All 16 rows complete. STATUS = 2 | Core writes CLEAR → S_IDLE |

⚠  S_PROG and S_RESET pulse reram_mode for exactly ONE cycle then move to their HOLD states. They do not hold mode high while waiting for CLEAR.

# Block 4 — imc_memory_block.sv  (Top Level)

## 4.1  What It Does

Pure structural wrapper. Contains no logic of its own — only instantiates the three submodules and wires them together. This is the only module an integrator needs to instantiate. All external connections (RISC-V core and DMA) attach here.

## 4.2  AXI4-Lite Ports (pass-through to imc_peripheral)

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| clk | 1 | IN | System clock |
| rst | 1 | IN | Synchronous active-high reset |
| s_axi_awaddr | 32 | IN | Write address — from RISC-V AXI master |
| s_axi_awvalid | 1 | IN | Write address valid |
| s_axi_awready | 1 | OUT | Write address ready |
| s_axi_wdata | 32 | IN | Write data |
| s_axi_wstrb | 4 | IN | Write byte strobes (passed through, ignored internally) |
| s_axi_wvalid | 1 | IN | Write data valid |
| s_axi_wready | 1 | OUT | Write data ready |
| s_axi_bresp | 2 | OUT | Write response (always OKAY) |
| s_axi_bvalid | 1 | OUT | Write response valid |
| s_axi_bready | 1 | IN | Core accepts write response |
| s_axi_araddr | 32 | IN | Read address |
| s_axi_arvalid | 1 | IN | Read address valid |
| s_axi_arready | 1 | OUT | Read address ready |
| s_axi_rdata | 32 | OUT | Read data (STATUS register) |
| s_axi_rresp | 2 | OUT | Read response (always OKAY) |
| s_axi_rvalid | 1 | OUT | Read data valid |
| s_axi_rready | 1 | IN | Core accepts read data |

## 4.3  DMA Handshake Ports

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| dma_trig_in | 1 | OUT | 1-cycle pulse — tells DMA to load next input row → ReRAM input_vec |
| dma_trig_out | 1 | OUT | 1-cycle pulse — tells DMA to save output row → SRAM |
| dma_src_addr | 15 | OUT | SRAM base word address of matrix A |
| dma_dst_addr | 15 | OUT | SRAM base word address for result matrix |
| dma_row | 4 | OUT | Current row (0–15) being processed |
| dma_done | 1 | IN | 1-cycle pulse from DMA — transfer complete |

## 4.4  SRAM Port A (DMA direct access)

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| dma_sram_addr | 15 | IN | SRAM word address DMA wants to read/write |
| dma_sram_wdata | 32 sgn | IN | Data to write into SRAM |
| dma_sram_we | 1 | IN | Write enable — 1=write, 0=read |
| dma_sram_rdata | 32 sgn | OUT | SRAM read data — valid 1 cycle after address presented |

## 4.5  ReRAM Data Ports (DMA direct access)

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| dma_reram_input_vec[16] | 32×16 sgn | IN | DMA writes 16-word input row here before each COMPUTE |
| dma_reram_output_vec[16] | 32×16 sgn | OUT | DMA reads 16-word result row here after each COMPUTE |

## 4.6  Internal Wires (connections between submodules)

| Wire | Width | From | To | Carries |
| --- | --- | --- | --- | --- |
| reram_mode | 2 | imc_peripheral | reram_array | Operation mode pulse |
| reram_prog_row | 4 | imc_peripheral | reram_array | Row to program |
| reram_prog_col | 4 | imc_peripheral | reram_array | Col to program |
| reram_prog_weight | 32 sgn | imc_peripheral | reram_array | Weight value |
| reram_done | 1 | reram_array | imc_peripheral | Compute complete pulse |
| reram_output_vec | 32×16 | reram_array | top-level port | Result vector (exposed as dma_reram_output_vec) |
| reram_input_vec | 32×16 | top-level port | reram_array | Input vector (driven by dma_reram_input_vec via assign) |

# 5 — How Everything Links Together

## 5.1  Full Connection Diagram

┌──────────────────────────────────┐

│         RISC-V PicoRV32          │

│         (AXI4 Master)            │

└────────────────┬─────────────────┘

AXI4-Lite    │  16 signals

(SW / LW)    │  awaddr/awvalid/awready

│  wdata/wstrb/wvalid/wready

│  bresp/bvalid/bready

│  araddr/arvalid/arready

│  rdata/rresp/rvalid/rready

┌────────────────────────────────────▼──────────────────────────────────┐

│                        imc_memory_block.sv                            │

│                          (top-level wrapper)                          │

│                                                                       │

│   ┌───────────────────────────────────────────────────────────────┐  │

│   │                    imc_peripheral.sv                          │  │

│   │               (AXI4-Lite slave + 9-state FSM)                 │  │

│   │                                                               │  │

│   │  reg_file: PROG_ROW/COL/WEIGHT, SRC_ADDR, DST_ADDR           │  │

│   │  triggers: TRIG_PROG, TRIG_COMPUTE, TRIG_RESET, CLEAR        │  │

│   │  status:   STATUS (readable by core)                          │  │

│   └──────┬────────────────────────────────────┬───────────────────┘  │

│          │ reram_mode[2]                       │ dma_trig_in          │

│          │ reram_prog_row[4]                   │ dma_trig_out         │

│          │ reram_prog_col[4]                   │ dma_src_addr[15]     │

│          │ reram_prog_weight[32]               │ dma_dst_addr[15]     │

│          │ reram_done ◄────────────            │ dma_row[4]           │

│   ┌──────▼──────────────────┐      │          │ dma_done ◄────────   │

│   │     reram_array.sv      │      │          │                  │   │

│   │   (16×16 MAC engine)    │      │          │                  │   │

│   │                         │      │          │                  │   │

│   │  W[16][16] weights      │      │          │                  │   │

│   │  input_vec ◄────────────┼──────┼──────────┼──────────────    │   │

│   │  output_vec ────────────┼──────┼──────────┼──────────────►   │   │

│   └─────────────────────────┘      │          │                  │   │

│                                    │          │                  │   │

│   ┌────────────────────────────────┼──────────┼──────────────────┼─┐ │

│   │          sram_block.sv          │          │                  │ │ │

│   │            (128KB)              │          │                  │ │ │

│   │  Port A ◄───────────────────────┼──────────┘                 │ │ │

│   │  (addr/wdata/we/rdata)          │                            │ │ │

│   └─────────────────────────────────┼────────────────────────────┘ │ │

└─────────────────────────────────────┼──────────────────────────────┘ │

│ DMA boundary                    │

┌───────────────────────▼─────────────────────────────────┘

│                     DMA

│  Reads SRAM → fills input_vec

│  Reads output_vec → writes SRAM

└──────────────────────────────────────────────────────────

## 5.2  Signal Flow — Programming a Weight

| Step | Who acts | What happens |
| --- | --- | --- |
| 1 | RISC-V core | SW to BASE+0x00 sets PROG_ROW |
| 2 | RISC-V core | SW to BASE+0x04 sets PROG_COL |
| 3 | RISC-V core | SW to BASE+0x08 sets PROG_WEIGHT |
| 4 | RISC-V core | SW to BASE+0x14 — TRIG_PROG |
| 5 | imc_peripheral | AXI write decoded → FSM moves S_IDLE → S_PROG |
| 6 | imc_peripheral | Pulses reram_mode=2'b01 for 1 cycle with prog_row/col/weight set |
| 7 | reram_array | W[prog_row][prog_col] <= prog_weight on that clock edge |
| 8 | imc_peripheral | STATUS set to 2 (done). Moves to S_PROG_HOLD |
| 9 | RISC-V core | LW from BASE+0x20 reads STATUS=2 |
| 10 | RISC-V core | SW to BASE+0x24 — CLEAR. FSM returns to S_IDLE |

## 5.3  Signal Flow — Full Matrix Multiply

| Step | Who acts | What happens |
| --- | --- | --- |
| 1 | RISC-V core | SW SRC_ADDR and DST_ADDR into peripheral registers |
| 2 | RISC-V core | SW to TRIG_COMPUTE |
| 3 | imc_peripheral | FSM → S_WAIT_DMA_IN. Pulses dma_trig_in, sets dma_row=0 |
| 4 | DMA | Reads 16 words from SRAM at src_addr+(row*16). Writes to reram_input_vec[0..15] |
| 5 | DMA | Pulses dma_done for 1 cycle |
| 6 | imc_peripheral | dma_done seen → FSM → S_COMPUTE. Pulses reram_mode=COMPUTE |
| 7 | reram_array | Latches input_vec into in_buf. MAC runs. 2 cycles later: output_vec valid, done=1 |
| 8 | imc_peripheral | reram_done seen → FSM → S_WAIT_DMA_OUT. Pulses dma_trig_out |
| 9 | DMA | Reads reram_output_vec[0..15]. Writes 16 words to SRAM at dst_addr+(row*16) |
| 10 | DMA | Pulses dma_done for 1 cycle |
| 11 | imc_peripheral | If row < 15: row_cnt++, dma_row++, pulse dma_trig_in → repeat from step 4 |
| 12 | imc_peripheral | If row = 15: FSM → S_DONE. STATUS = 2 |
| 13 | RISC-V core | LW STATUS → 2. SW CLEAR. Result matrix now in SRAM at DST_ADDR |

## 5.4  Who Owns What

| Resource | Owner | Others can... |
| --- | --- | --- |
| W[16][16] weights | reram_array | imc_peripheral writes via prog signals. Nobody reads directly. |
| reram_input_vec[16] | DMA | reram_array reads it every COMPUTE cycle. |
| reram_output_vec[16] | reram_array | DMA reads it after each compute to write back to SRAM. |
| SRAM mem[32768] | DMA (Port A) | imc_peripheral never touches SRAM. Core never touches SRAM. |
| STATUS register | imc_peripheral | RISC-V core reads it via AXI. |
| FSM state | imc_peripheral | Nobody — internal only. |

## 5.5  What the RISC-V Core Never Does

- The core never accesses SRAM directly — only the DMA reads and writes SRAM

- The core never drives input_vec — only the DMA loads data into reram_input_vec

- The core never reads output_vec — only the DMA transfers it to SRAM

- The core only ever writes to the 10 registers in imc_peripheral and polls STATUS

## 5.6  What the DMA Never Does

- The DMA never writes to the peripheral register map — no AXI transactions

- The DMA never controls reram_mode — only the peripheral does

- The DMA never knows about weights or the MAC — it only moves data

- The DMA acts only when dma_trig_in or dma_trig_out pulses HIGH